CANARY

Architecture & Summary — Locked, Current Spec

Supersedes all earlier CAN-FD / PUF / BTS7960 / 3-sensor versions of this document

# 1. System Overview

CANARY is a two-layer automotive intrusion detection system implemented on a 4-wheel-drive, skid-steer rover. Six ESP32 nodes exchange CAN-like messages over ESP-NOW (not physical CAN/CAN-FD hardware). Layer 1 verifies message authenticity and freshness using Ed25519 signatures and a BLAKE3 hash-tree freshness token. Layer 2 checks physical plausibility using an AI-aided Kalman filter fusing wheel-encoder and IMU data. A dedicated attacker node and a WiFi-to-ESP-NOW gateway node complete the six-node system.

# 2. Node Architecture (6 Nodes)

| Node | Role | Hardware | Signs messages? |
| --- | --- | --- | --- |
| Node 1 | Encoder ECU | ESP32 + motor encoders (4-wheel feedback) | Yes |
| Node 2 | IMU ECU | ESP32 + MPU6050 | Yes |
| Node 3 | Motor-Control ECU | ESP32 + 2x L298 + 4x DC geared motors | Executes approved commands only |
| Node 4 | Security / IDS ECU | ESP32 only — no sensor | Verifies Nodes 1, 2, 6 |
| Node 5 | Attacker | ESP32 only — no sensor, no valid key | No — that's the point |
| Node 6 | Gateway ECU | ESP32, bridges laptop (WiFi) → ESP-NOW | Yes |

# 3. Communication Architecture

Transport is ESP-NOW — a native ESP32-to-ESP32 wireless protocol, not physical CAN/CAN-FD. There is no CAN_H/CAN_L, no 120Ω termination, and no CAN transceiver hardware (MCP2518FD etc.) anywhere in this design. The application-layer message structure is deliberately designed to behave like a CAN frame (fixed fields, ID-based sender identification), so the security logic and message format are transport-agnostic — the same struct could later move to real CAN-FD hardware without changing Layer 1 or Layer 2 logic.

Documentation phrasing: “CAN-like simulated communication over ESP-NOW; CAN-FD physical-layer operation is not implemented.” Use this exact framing so the choice reads as deliberate, not as a misunderstanding of what CAN-FD requires.

Node 6 bridges an operator laptop (over WiFi) into the ESP-NOW mesh. The laptop→Node 6 WiFi hop is explicitly out of scope for CANARY's security guarantees — it is treated as a trusted operator channel. Node 6 itself is fully authenticated like every other legitimate node once it forwards a command onto ESP-NOW.

# 4. CANARY Message Format (90 bytes)

| Field | Size | Purpose |
| --- | --- | --- |
| sender_id | 1 byte | Which node claims to have sent this (1, 2, or 6 for legitimate traffic) |
| msg_type | 1 byte | wheel_speed / imu_data / motion_cmd / motor_ack |
| epoch | 4 bytes | Monotonic per-sender counter (not wall-clock time) — replay protection |
| payload | 4 bytes | The actual sensor reading or command value |
| freshness | 16 bytes | BLAKE3 hash-tree token for this epoch — second, independent freshness proof |
| signature | 64 bytes | Ed25519 signature over all fields above |
| Total | 90 bytes | Fits in a single ESP-NOW packet (250-byte limit) with 160 bytes to spare |

# 5. Layer 1 — Cryptographic Authentication & Freshness

Answers: “Who sent this, and is it fresh?”

- Ed25519 (via Monocypher, which uses BLAKE2b internally rather than SHA-512 — provisioning and firmware both use Monocypher consistently to avoid any interoperability mismatch).

- Each legitimate node (1, 2, 6) has one static Ed25519 keypair, generated once at provisioning. Node 4 stores only public keys; Node 5 (attacker) is provisioned with nothing.

- Freshness: a per-node monotonic epoch counter, checked first (cheap), plus a BLAKE3 hash-tree-derived freshness token (same tree construction as OSU's eCTF 2025 design, repurposed here as a second independent secret — not for confidentiality, since sensor values aren't secret, but so that forgery requires compromising two separate secrets rather than one).

- Verification order on Node 4, cheapest check first: (1) known sender_id, (2) epoch > last_seen[sender_id], (3) Ed25519 signature valid, (4) freshness token matches independently-recomputed tree value.

# 6. Layer 2 — Physical Plausibility (Sensor Fusion)

Answers: “Even if authenticated, does this match physical reality?”

- Encoder data (Node 1, all 4 wheels) and IMU data (Node 2, particularly Z-axis/yaw rate) are fused via an AI-aided Kalman filter on Node 4.

- Differential-drive kinematics check: expected_yaw_rate ≈ (v_right − v_left) / track_width, compared against the IMU's actual measured yaw rate. A large residual flags a physically implausible message — this replaces any dedicated steering-angle sensor, which was deliberately removed from the design (a potentiometer cannot survive being shaft-coupled to a continuously rotating wheel).

- This layer catches the one thing Layer 1 structurally cannot: a fully compromised node producing correctly-signed, fresh, but physically false messages.

# 7. Attack Handling

| Attack attempt | What happens | Caught by |
| --- | --- | --- |
| Fabricated message (no valid key) | Node 5 cannot produce a valid signature at all | Layer 1 — signature check |
| Verbatim replay of an old valid message | Signature is valid, but the epoch is old | Layer 1 — freshness check (epoch ≤ last_seen) |
| Replay with a manually bumped epoch | Epoch is part of the signed region, so editing it invalidates the signature | Layer 1 — signature check |
| Fully compromised node, correctly signed, fresh, but physically false | Message is cryptographically perfect but doesn't match real sensor behavior | Layer 2 — Kalman-filter residual check |

# 8. Physical Platform

- 4WD, skid-steer (differential wheel speed) — no mechanical steering axle, no steering servo.

- 2x L298 motor drivers (one per side) driving 4 DC geared motors independently — replaces the earlier BTS7960 choice, better matched to actual motor current draw.

- Chassis: 450mm (L) × 250mm (W), wheelbase 280mm, track width 180mm, wheel diameter 80mm, ground clearance ~20mm, two-tier deck layout.

- Power: 3S Li-ion battery (11.1V nominal) via 30A fuse and main switch, buck-converted to 5V for logic; single common ground across all 6 nodes.

# 9. Current Implementation Status

- Done and tested: CANARY message struct (90 bytes, confirmed via host compilation); Ed25519 sign/verify wrapper over Monocypher; provisioning tool generating real keypairs for Nodes 1, 2, 6; a 4-case test harness proving legitimate-message acceptance and rejection of forgery, tampered payload, and replay-with-edited-epoch — all passing on real cryptographic operations, run on-host (not yet on ESP32 hardware).

- Not yet started: BLAKE3 hash-tree freshness token (currently a zeroed placeholder field in the struct); ESP-NOW transport integration; Node 3/4/5/6 firmware; Layer 2 Kalman filter implementation; on-device (ESP32) testing.

# 10. Explicit Scope Boundaries

- The laptop→Node 6 WiFi link is a trusted operator channel, not defended by CANARY.

- Layer 2 cannot flag attacks on message types with no correlated physical sensor (e.g., a hypothetical non-motion command).

- This is a benchtop demonstration platform; a production deployment would consume sensor data already broadcast by existing vehicle ECUs (ABS, ESC) rather than requiring dedicated sensor hardware.